# Device Specifications & Hardware Requirements

Document Version: 1.0 Date: January 27, 2026 Project: SHUNCOM RULR IoT Platform Related Documents: 02-System Requirements

## 1. Overview of Supported Devices

The SHUNCOM RULR IoT Platform supports 7 device categories with specific communication protocols and configuration requirements.

### 1.1 Device Category Summary

| Device Type | Communication | Gateway Required | Quantity Support | Configuration Complexity |
| --- | --- | --- | --- | --- |
| Smart Gateway | Various protocols | No (Hub device) | 500 sub-devices each | High ⭐⭐⭐⭐ |
| Smart Light Controller | Zigbee/LoRa/NB-IoT/CAT.1 | Depends on type | 100,000+ per deployment | Medium ⭐⭐⭐ |
| Lighting Fixture | Via Controller | No (Logical device) | Unlimited | Low ⭐ |
| Lighting Pole | Via attached devices | No (Container) | Unlimited | Low ⭐ |
| Power Distribution | Via Gateway | Yes | As needed | Medium ⭐⭐ |
| Loop Control | Via Gateway | Yes | Gateway-dependent | Medium ⭐⭐⭐ |
| Smart Meter | Via Gateway | Yes | As needed | Medium ⭐⭐⭐ |

## 2. Smart Gateway Specifications

### 2.1 Device Overview

Role: Central hub for sub-device management and circuit control

Primary Functions: - Sub-device communication relay (Zigbee, Modbus, 485) - Loop/circuit control and monitoring - Local rule execution (autonomous operation) - Three-phase electricity monitoring - GPS time synchronization

### 2.2 Hardware Requirements

Processing Unit: - ARM Cortex-A7 or higher - Minimum 512MB RAM - 4GB+ Flash storage - Real-time clock (RTC) with battery backup

Communication Interfaces: - Ethernet: 10/100 Mbps (primary) - 4G LTE: Optional cellular backup - Zigbee: IEEE 802.15.4 coordinator - RS485: Modbus RTU support - GPS Module: Time sync and location

Power Requirements: - Input: AC 100-240V, 50/60Hz - Consumption: < 15W typical, < 30W peak - Backup battery: 24-hour operation

Environmental Specifications: - Operating temp: -40°C to +70°C - Storage temp: -40°C to +85°C - Humidity: 5% - 95% non-condensing - IP65 rating (outdoor installation)

### 2.3 Configuration Parameters

Mandatory Fields:

Device Name: "Gateway_[Location]_[Number]"
  Format: Alphanumeric, 50 characters max
  Example: "Gateway_MainSt_001"

Product Name: "Star Box Gateway"
  Type: Select from product catalog

Device Number: MAC Address
  Format: "AA:BB:CC:DD:EE:FF"
  Validation: Valid MAC format, unique per deployment

Optional Fields:

Associated Distribution Box: Link to power cabinet
Associated Circuit Control: Circuit controller ID
GPS Coordinates:
  Latitude: -90.0 to 90.0
  Longitude: -180.0 to 180.0
  Altitude: Meters above sea level
Parent Project: Project hierarchy assignment
Belonging Group: Device grouping

### 2.4 Circuit Configuration

Built-in Loop Controller: - Device Number: 0 (fixed) - Channel: 11 (fixed) - Protocol: GWLOOP (proprietary) - Circuits: Up to 10 circuits per gateway

Extended Loop Controllers: - Device Number: 4-255 (configurable) - Channel: 1 or 2 (physical connection) - Protocol: Modbus RTU - Expansion: Up to 252 external controllers

Circuit Operations:

Bulk Circuit Configuration:
  1. Add gateway
  2. Import circuit controllers
  3. Select gateway → Configure Circuits
  4. Generate circuits in batch
  5. Fine-tune individual circuits

Operations:
  - Open circuit (disconnect power)
  - Close circuit (connect power)
  - Schedule circuit operations
  - Monitor circuit status
  - Detect circuit faults

### 2.5 Gateway Operations

Administrative Functions:

Screen Password:
  Type: 6-digit numeric
  Purpose: Physical device access control
  Default: 123456 (must be changed)

Sub-device Synchronization:
  Function: Sync platform data to gateway
  Direction: Bidirectional
  Frequency: On-demand or scheduled
  Timeout: 60 seconds

Local Rule Management:
  Clear Rules: Remove all local rules
  Sync Rules: Download rules from platform
  Execute Rules: Autonomous rule processing

Three-phase Configuration:
  Transformer Ratio: Current transformer multiplier
  Voltage Ratio: Voltage transformer ratio
  Data Display: Multiplication factor

### 2.6 Monitoring Capabilities

Real-time Data Points: - Online/offline status - Sub-device count (active/total) - Circuit status (open/closed per circuit) - Three-phase voltage (V-A, V-B, V-C) - Three-phase current (I-A, I-B, I-C) - Active power (kW) - Reactive power (kVar) - Power factor - Total energy consumption (kWh) - Local time - GPS coordinates - Signal strength (if cellular) - Firmware version - Last update timestamp

## 3. Smart Light Controller Specifications

### 3.1 Protocol Matrix

| Protocol Type | Gateway Required | Network Activation | Range | Power | Complexity |
| --- | --- | --- | --- | --- | --- |
| Zigbee V3 | ✅ Yes | Automatic | 100m | Ultra-low | Medium ⭐⭐ |
| Dual-way Zigbee | ✅ Yes | Automatic | 100m | Ultra-low | Medium ⭐⭐ |
| NB-IoT | ❌ No | Carrier network | 10km+ | Low | High ⭐⭐⭐ |
| CAT.1 | ❌ No | Carrier network | 10km+ | Medium | High ⭐⭐⭐ |
| LoRa OTAA | ❌ No | Over-the-Air | 5-15km | Ultra-low | High ⭐⭐⭐⭐ |
| LoRa ABP | ❌ No | Personalization | 5-15km | Ultra-low | High ⭐⭐⭐⭐ |

### 3.2 Zigbee Light Controller (Pass-through)

Hardware Specifications:

Chip: Silicon Labs EFR32MG21 or equivalent
Protocol: Zigbee 3.0 IEEE 802.15.4
Frequency: 2.4 GHz
Network: Gateway coordinator required
Power: 3.3V DC, < 1W

Configuration Requirements:

Mandatory:
  Device Name: "Light_Controller_[ID]"
  Product: "Zigbee_V3 Light Controller"
  Gateway: [Select from gateway list]
  Light Pole: [Select from pole list]
  Associated Luminaires: [CRITICAL - Must select]

Optional:
  Installation Location: Physical address
  GPS Coordinates: Lat/Long
  Group Assignment: Device group

Control Capabilities: - On/Off control - Dimming: 0-100% (1% increments) - Color temperature: 2700K-6500K (if supported) - Scene control - Schedule execution - Local rule processing

Monitoring Data:

Electrical Parameters:
  - Voltage (V)
  - Current (A)
  - Active Power (W)
  - Power Factor
  - Energy Consumption (kWh)
  - Frequency (Hz)

Environmental:
  - Illuminance (Lux)
  - Color Temperature (K)

Network:
  - Signal Strength (dBm)
  - Link Quality Indicator (LQI)
  - Parent Node ID
  - Hop Count

Status:
  - Online/Offline
  - Lamp Status (On/Off/Dimming %)
  - Run Time (hours)
  - Local Time
  - Firmware Version

### 3.3 NB-IoT Light Controller (Direct)

Hardware Specifications:

Modem: Quectel BC95/BC25 or equivalent
Protocol: NB-IoT (LTE Cat-NB1/NB2)
Frequency Bands: B3, B5, B8, B20 (regional)
SIM Card: Nano SIM (carrier activation required)
Power: 12V DC, < 3W typical, < 5W peak

Network Requirements:

Carrier Network:
  - Coverage verification required
  - Signal strength: > -110 dBm
  - SIM card activation before deployment
  - APN configuration per carrier

Platform Communication:
  - Protocol: MQTT over TCP
  - Encryption: TLS 1.2
  - Heartbeat: Every 60 seconds
  - Data reporting: Event-driven + periodic (5 min)

Configuration Requirements:

Mandatory:
  Device Name: "NB_Controller_[ID]"
  Product: "Cat.1-V3 Light Controller"
  Device Address: Unique device identifier (IMEI)
  Associated Luminaires: [CRITICAL - Must select]
  SIM ICCID: SIM card identifier

Optional:
  APN Settings: Carrier-specific
  Server Address: Custom MQTT broker
  Reporting Interval: 1-60 minutes

Advantages: - No gateway required (direct cloud connection) - Long-range communication (10km+) - Deep penetration (underground, indoor) - Low power consumption

Limitations: - Carrier network dependency - Monthly data charges - Activation complexity - Higher latency vs. Zigbee

### 3.4 LoRa Light Controller (LoRaWAN)

Hardware Specifications:

Chip: Semtech SX1276/SX1278
Protocol: LoRaWAN 1.0.3 / 1.1
Frequency Bands:
  - EU868: 863-870 MHz
  - US915: 902-928 MHz
  - AS923: 915-928 MHz
  - CN470: 470-510 MHz
Power: 3.6V DC, < 0.5W

OTAA Configuration (Recommended):

Activation: Over-the-Air Activation
Security: Session keys generated dynamically

Required Parameters:
  DEVEUI: Device unique identifier (64-bit)
    Format: "70B3D57ED005XXXX"
    Source: Device label or firmware

  DEV_PROFILE: LoRaWAN device profile
    Example: "Class A, OTAA"

  APPEUI: Application identifier (64-bit)
    Format: "70B3D57ED0000000"
    Provided by: Network administrator

  APPKEY: Application key (128-bit)
    Format: "2B7E151628AED2A6ABF7158809CF4F3C"
    Security: Store securely, never expose

ABP Configuration (Alternative):

Activation: Activation by Personalization
Security: Pre-configured session keys

Required Parameters:
  DEVEUI: Device unique identifier (64-bit)

  DEV_PROFILE: LoRaWAN device profile

  DEVADDR: Device address (32-bit)
    Format: "26011XXX"
    Unique: Within network

  APPSKEY: Application session key (128-bit)
    Format: "2B7E151628AED2A6ABF7158809CF4F3C"

  NWKSKEY: Network session key (128-bit)
    Format: "3C4F0C8D09F71584A6D2AE2816157E2B"

LoRaWAN Network Requirements:

Gateway Coverage:
  - LoRaWAN gateway within range
  - Gateway density: 1 per 2-5 km² (urban)
  - Line-of-sight preferred

Network Server:
  - ChirpStack / TTN / AWS IoT Core
  - Device registration required
  - Application integration configured

Data Rate:
  - DR0-DR5 (adaptive)
  - Typical: 5-50 kbps
  - Duty cycle: < 1% (EU), < 100% (US)

Advantages: - Ultra-low power (10+ years battery) - Long range (5-15km) - No SIM card costs - No gateway required (if LoRaWAN network exists)

Limitations: - LoRaWAN infrastructure required - Low data rate - Higher configuration complexity - Duty cycle restrictions

## 4. Lighting Fixture Specifications

### 4.1 Logical Device Role

Purpose: Represent physical lights in the system

Critical Requirement: Must exist before light controller association

Relationship: 1-to-1 mapping with light controller

### 4.2 Configuration Parameters

Basic Information:
  Device Name: "LED_Fixture_[Location]_[Number]"
    Example: "LED_Fixture_MainSt_045"

  Product Name: "High-Power LED Streetlight"
    Types:
      - LED Streetlight (100W-400W)
      - LED Flood Light
      - LED Garden Light
      - Decorative Light

  Project Assignment: [Select project]
  Group Assignment: [Select group]

Physical Specifications:
  Wattage: Rated power (W)
  Voltage: Operating voltage (V)
  Luminous Flux: Light output (Lumens)
  Color Temperature: 2700K-6500K
  CRI: Color Rendering Index (>80)
  IP Rating: IP65-IP67

Installation Data:
  Installation Date: YYYY-MM-DD
  Warranty Expiry: YYYY-MM-DD
  Maintenance Interval: Days

### 4.3 Association Workflow

graph LR
    A[Create Fixture] --> C[Association]
    B[Create Controller] --> C
    C --> D[Fixture-Controller Link]
    D --> E[Light Control Enabled]

Flexible Creation Order: 1. ✅ Fixture first → Controller later → Associate 2. ✅ Controller first → Fixture later → Associate 3. ✅ Both created → Associate after

Warning: Without association, system displays “Lamp uncontrollable in list”

## 5. Lighting Pole Specifications

### 5.1 Infrastructure Container

Purpose: Organize devices by physical location

Contains: - Light controllers - Lighting fixtures - Sensors (illuminance, motion) - Smart meters - Cameras (optional)

### 5.2 Configuration

Basic Information:
  Device Name: "Pole_[Street]_[Number]"
    Example: "Pole_MainSt_045"

  Product: "Standard Street Light Pole"
    Types:
      - Single-arm pole
      - Double-arm pole
      - High-mast pole
      - Decorative pole

  Location: Physical address or GPS

Physical Specifications:
  Height: Meters
  Material: Steel/Aluminum/Concrete
  Foundation Type: Direct burial/Base plate

Tracking:
  Subordinate Devices: Auto-calculated count
  Associated Equipment: Auto-linked
  Installation Date: YYYY-MM-DD

## 6. Power Distribution Control Specifications

### 6.1 Smart Distribution Cabinet

Purpose: Monitor and control electrical circuits

Integration: Works with gateways and circuit controllers

### 6.2 Hardware Requirements

Electrical:
  Input Voltage: 3-phase AC 380V/220V
  Rated Current: 63A-630A per circuit
  Frequency: 50/60 Hz
  Protection: MCB, RCD, SPD

Monitoring:
  Voltage: Per phase
  Current: Per phase
  Power: Active, reactive, apparent
  Energy: Cumulative kWh
  Temperature: Cabinet internal temp

Communication:
  Protocol: Modbus RTU/TCP
  Interface: RS485/Ethernet
  Gateway: Required for platform integration

### 6.3 Configuration

Device Name: "Distribution_Box_[Area]_[ID]"
Product: "Smart Distribution Cabinet"
Project: [Infrastructure project]
Group: "Power Distribution"

Capabilities:
  - Individual circuit monitoring
  - Load balancing
  - Emergency disconnect
  - Scheduled operations
  - Fault detection
  - Energy reporting

## 7. Loop Control Specifications

### 7.1 Built-in Loop Controller

Integration: Inside gateway (Star Box models)

Specifications:

Device Number: 0 (Fixed, auto-assigned)
Channel: 11 (Fixed)
Protocol: GWLOOP (Proprietary)
Gateway: Parent gateway (required)

Circuits: Up to 10 independent circuits
Operations:
  - Open circuit (disconnect)
  - Close circuit (connect)
  - Schedule control
  - Manual override

Status Monitoring:
  - Circuit state (open/closed)
  - Load current per circuit
  - Fault detection

### 7.2 Extended Loop Controller

Integration: External Modbus device

Specifications:

Device Number: 4-255 (User-configured)
  Unique: Within gateway

Channel: 1 or 2
  Mapping: Physical RS485 connection
  Verification: Check physical wiring

Protocol: Modbus RTU
  Baud Rate: 9600/19200 bps
  Parity: None/Even/Odd
  Stop Bits: 1/2

Gateway: Parent gateway (required)
Expansion: Up to 252 controllers per gateway

Configuration Matrix:

| Type | Device Number | Channel | Protocol | Circuits |
| --- | --- | --- | --- | --- |
| Built-in | 0 (fixed) | 11 (fixed) | GWLOOP | 10 |
| Extended | 4-255 | 1 or 2 | Modbus | 2-10 per unit |

## 8. Smart Meter Specifications

### 8.1 485 Three-Phase Electricity Meter

Purpose: Detailed energy consumption monitoring

Protocol: DLT645-07 (Chinese national standard)

### 8.2 Hardware Requirements

Electrical:
  Voltage: 3-phase 4-wire, 380V/220V
  Current: 5A-100A (direct) or via CT
  Frequency: 50 Hz
  Accuracy Class: 1.0 or 0.5S

Measurement:
  Active Energy: kWh (import/export)
  Reactive Energy: kVarh
  Voltage: Per phase (V)
  Current: Per phase (A)
  Power: Active, reactive, apparent (kW, kVar, kVA)
  Power Factor: Cosφ
  Frequency: Hz

Communication:
  Interface: RS485
  Protocol: DLT645-07
  Baud Rate: 1200/2400/4800/9600 bps
  Meter Address: 12-digit unique ID

### 8.3 Configuration Requirements

Mandatory:
  Device Name: "Smart_Meter_[Location]_[ID]"
  Device Number: "12-digit meter address"
    Format: "XXXXXXXXXXXX"
    Source: Meter display or label

  Gateway: [Required association]
  Channel: 2 (Default for meters on gateway)

Optional:
  Transformer Ratio: Current transformer multiplier
    Example: 100/5 = 20
    Purpose: Multiply readings for actual consumption

  Data Display: Multiplication factor
    Purpose: Adjust displayed values

Advanced:
  Three-phase Configuration:
    - Phase A association
    - Phase B association
    - Phase C association
    - Total calculation method

### 8.4 Device Synchronization

Sync Operations:

Sync Meter Info to Gateway:
  Function: Update meter configuration
  Direction: Platform → Gateway → Meter
  Data: Address, ratio, display settings

Update Parameters:
  Function: Configure meter settings
  Parameters: Reporting interval, alarms, thresholds

Read Meter Data:
  Function: Retrieve current readings
  Frequency: 5-60 minutes (configurable)
  Data Points: All electrical parameters

## 9. Common Configuration Errors & Solutions

### 9.1 Fixture Association Missing

Problem: Light controller created without fixture Symptom: “Lamp status blank in list” or “Lamp uncontrollable” Solution: 1. Create lighting fixture 2. Associate with controller 3. Refresh device list 4. Verify control operations

### 9.2 Gateway Association Missing

Problem: Sub-device created without gateway Symptom: Device always shows offline Solution: 1. Verify gateway exists and is online 2. Associate sub-device with gateway 3. Trigger device synchronization 4. Check gateway sub-device list

### 9.3 Coordinate Data Missing

Problem: GPS coordinates not configured Symptom: Sunrise/sunset rules fail to execute Solution: 1. Add latitude/longitude to device 2. Verify coordinate accuracy (Google Maps) 3. Update existing rules 4. Test rule execution

### 9.4 Time Zone Mismatch

Problem: Incorrect time zone setting Symptom: Rules execute at wrong times Solution: 1. Navigate to My Account → Preferences 2. Select correct time zone 3. Verify device local time updates 4. Resync local rules to devices 5. Test scheduled operations

## 10. Performance Optimization Guidelines

### 10.1 Batch Operations

Maximum Limits:
  Import/Export: 5,000 devices per operation
  Reason: UI performance, database transaction size

Template Usage:
  - Use device-specific templates
  - Validate data before import
  - Fix errors incrementally
  - Monitor import progress

Best Practices:
  - Split large imports (>5000) into multiple batches
  - Import during low-traffic periods
  - Validate sample before full import
  - Keep backup of source data

### 10.2 Device Limits

Recommended Limits:
  Devices per Project: <1,000 for optimal UI performance
  Gateway Sub-devices: <50 per gateway
  Multicast Group Size: <200 devices

Scaling Considerations:
  - Use sub-projects for large deployments
  - Distribute devices across multiple gateways
  - Limit real-time dashboard widgets
  - Use filtered views instead of "All Devices"

## 11. Device Procurement Checklist

### 11.1 Smart Gateway Procurement

Specifications to Verify:
  ✅ ARM processor, 512MB+ RAM
  ✅ Zigbee coordinator support
  ✅ RS485 interface (Modbus RTU)
  ✅ GPS module for time sync
  ✅ 4G LTE backup (optional)
  ✅ IP65 outdoor rating
  ✅ -40°C to +70°C operating temp
  ✅ Firmware upgradeable (OTA)

Quantity Estimation:
  - 1 gateway per 30-50 Zigbee devices
  - 1 gateway per electrical distribution area
  - +20% spare units for failures

### 11.2 Light Controller Procurement

Protocol Selection Criteria:

Zigbee V3:
  ✅ Use when: Gateway infrastructure exists
  ✅ Advantages: Low cost, low power, easy setup
  ❌ Limitations: Gateway dependent, 100m range

NB-IoT/CAT.1:
  ✅ Use when: Wide-area coverage needed, no gateways
  ✅ Advantages: Long range, no infrastructure
  ❌ Limitations: SIM costs, carrier dependency

LoRa:
  ✅ Use when: LoRaWAN network available, battery power
  ✅ Advantages: Ultra-low power, long range
  ❌ Limitations: Complex setup, LoRaWAN infrastructure

Quantity Estimation:
  - 1 controller per light (1-to-1 mapping)
  - +10% spare units for failures
  - Consider protocol mix for resilience

### 11.3 Supporting Equipment

Lighting Fixtures:
  - Match controller quantity (1-to-1)
  - Verify voltage compatibility
  - Check dimming support (0-10V, PWM)
  - IP65+ rating for outdoor

Lighting Poles:
  - Infrastructure survey required
  - Existing pole mapping
  - New pole specifications

Smart Meters:
  - 1 meter per distribution cabinet
  - 1 meter per 10-20 lighting circuits
  - DLT645-07 protocol compliance
  - CT ratio calculation

Power Distribution:
  - Smart cabinet per lighting zone
  - MCB sizing per circuit load
  - RCD protection
  - SPD surge protection

Next Document: 04-Technical Architecture

## Document Revision History

| Version | Date | Changes | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-01-27 | Initial device specifications | AI Assistant |